Illustrative example on the CWRU bearing dataset

To address the reviewer’s suggestion, a minimal illustrative example is provided to demonstrate the complete execution procedure of the proposed approach. All parameter settings and operational steps are explicitly described to ensure replicability.

Dataset partition

The complete CWRU bearing dataset contains consists of 1181 sets of data, which are divided into a training dataset Dtrain with 945 sets of data and a testing dataset Dtest with 236 sets of data. Table 1 presents the statistics of the dataset.

Table 1. Statistics of the CWRU bearing dataset.

| Dataset | No. | Input |  |  |  | Output |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | x1 | x2 | x3 | x4 | y1 |
| Dtrain | 1 | 6.5796E-02 | 4.3329E-01 | -1.6675E-02 | 2.9011E+00 | 5.2862E-02 |
|  | 2 | 6.7278E-02 | 3.9846E-01 | -1.6571E-01 | 2.7075E+00 | 5.4491E-02 |
|  | 3 | 6.5342E-02 | 4.0304E-01 | -1.5899E-01 | 2.8869E+00 | 5.3042E-02 |
|  | … | … | … | … | … | … |
|  | 24 | 6.7320E-02 | 4.3225E-01 | -1.0523E-01 | 3.1748E+00 | 5.3643E-02 |
|  | 25 | 6.4224E-02 | 4.4060E-01 | -1.0473E-01 | 2.8087E+00 | 5.2319E-02 |
|  | … | … | … | … | … | … |
|  | 90 | 7.2806E-02 | 4.5770E-01 | -2.3654E-02 | 2.8664E+00 | 5.8170E-02 |
|  | … | … | … | … | … | … |
|  | 110 | 6.7363E-02 | 4.9463E-01 | -1.1782E-01 | 3.5364E+00 | 5.2633E-02 |
|  | … | … | … | … | … | … |
|  | 945 | 6.6869E-02 | 4.2245E-01 | -9.6148E-02 | 2.9123E+00 | 5.3095E-02 |
| Dtest | 946 | 7.0562E-02 | 5.0610E-01 | -2.8944E-01 | 3.2171E+00 | 5.6567E-02 |
|  | … | … | … | … | … | … |
|  | 1181 | 7.1695E-02 | 3.9866E-01 | -5.5033E-02 | 2.6490E+00 | 5.7938E-02 |

Multiple model construction

According to Step 1 in Section 3.2, three models are constructed, namely BPNN, GPR and KNN, and the parameter settings of three models are shown in the Table 2. Table 3 shows the prediction results on the training dataset Dtrain.

Table 2. Parameter settings of the three models.

| Model | Parameter settings |
| --- | --- |
| BPNN | Hidden Layer=[3, 3], Activation Function=tansig, Max Epochs=1000, Early Stopping=Enabled |
| GPR | Kernel Type=ARD-SE, Fitting Method =Exact, Max Optimizer Iterations=1000 |
| KNN | Number of Neighbors=5, Distance Metric=Cosine, Weighting Method= Equal Distance |

Table 3. Predictive outputs of the three models on the training dataset

| Data No. | Model |  |  | Real output |
| --- | --- | --- | --- | --- |
|  | BPNN | GPR | KNN |  |
| 1 | 5.2813E-02 | 5.2798E-02 | 5.2701E-02 | 5.2862E-02 |
| 2 | 5.4735E-02 | 5.4693E-02 | 5.4676E-02 | 5.4491E-02 |
| 3 | 5.2679E-02 | 5.2691E-02 | 5.3200E-02 | 5.3042E-02 |
| … | … | … | … | … |
| 945 | 5.3737E-02 | 5.3744E-02 | 5.3533E-02 | 5.3095E-02 |

Discrepancy identification

According to Step 2 in Section 3.3, discrepancy detection is conducted based on the relative absolute errors of each model. Specifically, for each model, data whose prediction errors fall within the top 25% (ranked from high to low) are identified as high-error data. Table 4 shows the process of discrepancy detection.

Table 4 A numerical example of discrepancy detection

| Data No. | Model |  |  |  |  |  | Discrepancy identification result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BPNN |  | GPR |  | KNN |  |  |
|  | Error | Rank* | Error | Rank | Error | Rank |  |
| 1 | 4.8864E-05 | 750 | 6.3853E-05 | 723 | 1.6072E-04 | 422 | Congruent data |
| 2 | 2.4427E-04 | 206* | 2.0188E-04 | 310 | 1.8462E-04 | 368 | Ω(BPNN) |
| 3 | 3.6303E-04 | 65 | 3.5154E-04 | 96 | 1.5745E-04 | 430 | Ω(BPNN, GPR) |
| … | … | … | … | … | … | … | … |
| 24 | 2.1422E-04 | 260 | 1.8123E-04 | 350 | 6.4339E-04 | 29 | Ω(KNN) |
| 25 | 2.2897E-04 | 244 | 2.7393E-04 | 177 | 1.5604E-04 | 435 | Ω(GPR) |
| … | … | … | … | … | … | … | … |
| 90 | 2.3745E-04 | 240 | 3.1554E-04 | 128 | 3.2244E-04 | 167 | Ω(GPR, KNN) |
| … | … | … | … | … | … | … | … |
| 110 | 2.7147E-04 | 163 | 2.2089E-04 | 268 | 4.2625E-04 | 88 | Ω(BPNN, KNN) |
| … | … | … | … | … | … | … | … |
| 945 | 6.4283E-04 | 2 | 6.4937E-04 | 2 | 4.3872E-04 | 77 | Ω(BPNN, GPR, KNN) |

*Rank: Error ranking from high to low.

*Number: The top 25% error of each model by clustering.

*206: The data is identified as high-error data.

Table 5 presents the MAEs of each model within different cross areas. Ω(BPNN, GPR, KNN) and Ω(GPR, KNN) are selected for normalized weight calculation owing to the following reasons:

(1) Ω(BPNN, GPR, KNN) is primarily selected for the discrepancy rectification since all three models demonstrate the highest MAEs.

(2) Ω(GPR, KNN) is incorporated because the MAEs remain higher than those in Ω(BPNN, GPR) and Ω(BPNN, KNN).

Table 5 MAEs comparison of cross areas composed of BPNN, GPR, and KNN.

| Cross area | MAE |  |  |
| --- | --- | --- | --- |
|  | BPNN | GPR | KNN |
| Ω(BPNN, GPR, KNN) | 5.4683E-02 | 6.5474E-02 | 5.4701E-02 |
| Ω(BPNN, GPR) | 5.4701E-02 | 6.5471E-02 | / |
| Ω(BPNN, KNN) | 5.4630E-02 | / | 5.4564E-02 |
| Ω(GPR, KNN) | / | 6.5164E-02 | 5.5980E-02 |
| Ω(BPNN) | 4.9284E-02 | / | / |
| Ω(GPR) | / | 4.5775E-02 | / |
| Ω(KNN) | / | / | 4.7591E-02 |

Discrepancy rectification

The rectification process is carried out by calculating model weights according to the MAEs summarized in Table 5. According to Step 3 in Section 3.4, cross area exhibiting stronger discrepancy are reconstructed through the model weight, where the weight magnitude is inversely related to the corresponding MAE.

(1) Ω(BPNN, GPR, KNN) reconstruction: This cross area reflects the highest errors identified by all three models. According to Eq. (3) in Section 3.4, the weights are calculated by the following Eq. (1),

|  |  | (1) |
| --- | --- | --- |

where the weights are then normalized by the following Eq. (2),

|  |  | (2) |
| --- | --- | --- |

According to Step 4 in Section 3.4, the new output of the 945th data under Ω(BPNN, GPR, KNN) is calculated by the following Eq. (3),

|  |  | (3) |
| --- | --- | --- |

where yBPNN,945, yGPR,945, and yKNN,945, denotes the predicted output of the respective models.

(2) Ω(GPR, KNN) reconstruction: For the Ω(GPR, KNN), only the predictions from GPR and KNN are used for weight calculation by the following Eq. (5),

|  |  | (5) |
| --- | --- | --- |

where the weights are also normalized, resulting in  (0.4621) and  (0.5379). Then, the new output of the 90th data under the Ω(GPR, KNN) is calculated by the following Eq. (6),

|  |  | (6) |
| --- | --- | --- |

Following the discrepancy rectification strategy, the discrepant data are reconstructed. These data, along with the congruent data, are merged to generate a new training dataset Dnew.

Model optimization

According to Step 5 in Section 3.5, the BRB model is trained using the new training dataset. Then, the predictions are generated on the testing dataset according to Step 6 in Section 3.6. The model’s predictive performance is evaluated by calculating the MAE between the predicted outputs by the distilled BRB and the original outputs.

Fig. 1 shows the comparison of the prediction results on the test set by the BRBs trained on the original training set Dtrain and the new training set Dnew after discrepancy rectification, respectively. The BRB trained on the new training set reduces the MAE from 8.0452E-04 (trained on the original training dataset) to 6.7570E-04 with a decrease of 16.01%.

Fig. 1. Comparison of outputs and absolute errors on the testing dataset before and after discrepancy rectification: (a) Absolute errors, and (b) Outputs.